.onlineHelp  ENVISION "Display Control" SPECTRUM

.section 1 "Display Capabilities"

.paragraph

The "Display" subform allows you to normalize the image colormap.  It

also lets you control which bands or functions of bands are displayed

as Red, Green, and Blue.  In addition, you can use this subform to

change the names assigned to each band.

.paragraph

.section 2  "Colormap Normalization"

Spectrum allows you to set two different parameters which control

colormap normalization: the type of normalization (local or global),

and the method used for normalization.  Employing various combinations

of values for the two parameters results in widely varied colormap

display.

.section 3 "Type of Normalization"

.include $DESIGN/manual/shared/envision/display/norm_type.doc

.section 3 "Normalization Method"

.include $DESIGN/manual/shared/envision/display/norm_method.doc

.section 2  "Changing the Bands Displayed as Red, Green, Blue"

.include $DESIGN/manual/shared/envision/display/mapcol.doc

.section 2  "Applying Functions of Bands (Transformations)"

.paragraph

There are a variety of "indices" used to translate spectral

information to particular types of landscape features.  For example,

you might define the red intensity as being dictated by the value in

the fifth band, green as computed with the value (band4 -

band3)/(band4 + band3), and blue as dictated by the second band.  In

this case, the "RED," "GREEN," and "BLUE" string selections may be

used to define the function that will produce the values used as red,

green, and blue.  Functions must have \fIonly one\fP variable \fIM\fP;

following each \fIM\fP must be a number \fIstarting at zero\fP and

ranging to \fIN-1\fP, where \fIN\fP is the number of bands available

in the image.  In our example, the equations would be:

.sp

Red: M4

.br

Green: (M3-M2)/(M3+M2)

.br

Blue: M1

.paragraph

Equation evaluation follows the standard rules of precedence:

evaluation proceeds from left to right, and use of parentheses is

fully supported.

.sp

As implied by the above example, the red, green, or blue functions

can be set to represent the band that specifies the red, green, or

blue pixel values in the image.  Supply \fIMx\fP as the "function"

where "x" is the band number - 1 (remember that numbering begins at

0).  This is equivalent to using the buttons labeled "Change Red Map

Col," "Change Green Map Col," and "Change Blue Map Col."

.paragraph

You can also set the "RED," "GREEN," and "BLUE" functions to a

constant.  For example, if you wanted the red values of all the pixels

in the image to be 200, you could enter "200" in the "RED" function.

.section 2 "Color Transformations"

.paragraph

"Transformations," or sets of equations for red, green, and blue,

formulated specifically to get particular types of landscape features,

can also be supplied in a Display Environment File for convenient

access.  For transformations that are used often, the Display

Environment File is recommended over manual entry from the Spectrum

GUI.  For more information about the Display Environment File, see the

section on the Files subform for Spectrum.  When transformations are

supplied in the Display Environment file, the "Transformations" button

on the Display subform will be activated.

.paragraph

Clicking on the "Transformations" button will pop up a list of the

transformations defined in the Display Environment file.  Select the

desired transformation from the list and it will be immediately

applied to the image.  Note that this is a more convenient method

of applying frequently used transformations than entering each

function for red, green, and blue by hand.