.onlineHelp ENVISION "Windowing/Thresholding" EDITIMAGE

.indexEntry regular "clipping" ""

.indexEntry regular "thresholding" ""

.indexEntry regular "pixel clipping" ""

.indexEntry regular "pixel thresholding" ""

.indexEntry regular "pixel clipping" "image"

.indexEntry regular "pixel thresholding" "image"

.glossary "threshold object" "A threshold visual object provides a mechanism with which an image may be clipped or thresholded."

.section 1 "Thresholding"

.paragraph

The threshold feature of Editimage provides a mechanism with which you

may perform pixel thresholding, pixel clipping, or window contrast

enhancement operations on the currently displayed image. Set the

toggle above the threshold palette to "Pixel Clipping," "Pixel

Thresholding," or "Window Contrast Enhancement" depending on the type

of thresholding operation that you wish to perform.

.paragraph

Note that the thresholding feature will be disabled for RGB images

when Editimage is displayed on an 8-bit screen.  This is because a

mapping transformation is used in order to convert the RGB image into

an 8-bit representation.  Because of this transformation, thresholding

an RGB image on an 8-bit screen does not make sense; any changes to

the colormap made via thresholding would alter the 24-bit to 8-bit

transformation (a 3-3-2 RGB map is used) rather than the RGB values

themselves, producing an undefined result.  To reduce confusion, the

Threshold subform button is deactivated, and the Threshold subform

closed if it is open, when an RGB image is entered to Editimage on an

8-bit screen.

.section 2 "Pixel Clipping"

.paragraph

Pixel clipping is used to mask out certain parts of the image so that

only those pixels that fall within a specified range are observed.

Pixel clipping works by allowing a windowed region of the original

pixel values to be displayed.  The user selects the pixel value of the

masked region; all values outside the windowed region will be set to

this masked region pixel value.  If the masked region pixel value does

not exist in the colormap, the corresponding grey level value will be

used.

.paragraph

When the "Invert Region?" logical is set to "False," the original

pixel values of the image will be displayed \fIinside\fP the region

defined by the lower and upper scroll bars; this can be thought of as

a bandpass filter.  When the "Invert Region?" logical is set to

"True," the original pixel values will be displayed \fIoutside\fP the

region defined by the lower and upper scroll bars; this can be thought

of as a bandreject filter.

.section 2 "Pixel Thresholding"

.paragraph

Pixel thresholding is used to perform real-time image thresholding.

It generates a binary (or 2-pixel value) image.  Pixel thresholding

works by assigning a non-zero pixel value to the selected region; the

rest of the image will be turned black.

.paragraph

When the "Invert Region?" logical is set to "False," the pixels in the

image \fIinside\fP the region defined by the lower and upper scroll

bars will be displayed as white, and those outside the region will be

displayed as black.  When the "Invert Region?" logical is set to

"True," the opposite display is used; that is, those values

\fIoutside\fP the region defined by the lower and upper scroll bars

will be displayed as white, and those inside the region will be

displayed as black.

.section 2 "Window Contrast Enhancement"

.paragraph

Windowed thresholding is used to increase the contrast of the image in

a particular pixel range while masking out the other pixels in the

image.  Use the "Lower," "Upper," and "Range" scrollbars to

interactively specify the region of pixel values that will have their

contrast enhanced; all pixels outside the range will appear in black.

The pixel values inside the specified range will have their pixel

values normalized to cover the entire range of pixels in the image;

thus, this operation can be thought of as a histogram stretch.

.paragraph

When "Window Contrast Enhancement" is active, the "Invert Region?"

button is disabled.

.section 2 "Reset (undo thresholding)"

.paragraph

This button resets the lower and upper values of the threshold range

to the original minimum and maximum values of the image being

displayed.  This has the effect of undoing any prior thresholding

operations and setting the image back to its "un-thresholded" state.