Package:        audio/speech/gsm

Description:	GSM 06.10 13 kbit/s RPE/LTP speech compressor

Version:	1.0p2

Notes:

The Communications and Operating Systems Research Group (KBS) at the

Technische Universitaet Berlin is currently working on a set of UNIX-based

tools for computer-mediated telecooperation that will be made freely

available.

As part of this effort we are publishing an implementation of the European

GSM 06.10 provisional standard for full-rate speech transcoding, prI-ETS 300

036, which uses RPE/LTP (residual pulse excitation/long term prediction)

coding at 13 kbit/s.

GSM 06.10 compresses frames of 160 13-bit samples (8 kHz sampling rate, i.e.

a frame rate of 50 Hz) into 260 bits; for compatibility with typical UNIX

applications, our implementation turns frames of 160 16-bit linear samples

into 33-byte frames (1650 Bytes/s).  The quality of the algorithm is good

enough for reliable speaker recognition; even music often survives

transcoding in recognizable form (given the bandwidth limitations of 8 kHz

sampling rate).

The interfaces offered are a front end modelled after compress(1), and a

library API.  Compression and decompression run faster than realtime on most

SPARCstations.  The implementation has been verified against the ETSI

standard test patterns.

.../README

Language(s):    C

Requirements:   ?

Origin:

tub.cs.tu-berlin.de:/pub/tubmik

*

Jutta Degener, jutta@cs.tu-berlin.de

Carsten Bormann, cabo@cs.tu-berlin.de

See Also:	?

Restrictions:

See .../COPYRIGHT

References:

?